Lite3 Pilot console — setup guide

Covers: command buttons, two-joystick driving and camera setup. Applies to the web console (port 8123) and its config.json.

1. Commands (buttons)

1.1 How the buttons are organised

Buttons are grouped exactly as configured:

•	
      Top row: 🐕 STAND · 🪑 SIT · 👋 HELLO · 🕺 DANCE · 🔄 TURN OVER (built-in pose buttons)

•	
      Recovery (from upside down): ↺ Recover left · ↻ Recover right

•	
      Poses & actions: 🌀 Twist · 🌪️ Twist jump · 🤸 Backflip · 🦘 Long jump (plus Hello/Dance/Turn over)

•	
      Modes: 🎮 Manual mode · 🕹️ Move mode

•	
      Gaits: 🐢 Slow · 🚶 Medium · ⚡ Fast · 🐛 Crawl

1.2 The posture rule (very important)

•	
      STAND and SIT are one and the same command (0x21010202) — a toggle. Sending it while already standing makes the robot sit, and vice versa.

•	
      Actions that need a specific pose declare it ("posture": "sit" or "stand"). The console only re-poses when it knows the state is wrong; with "force_posture": true it re-poses even when the state is unknown (used for rolls/jumps).

•	
      The posture tracker resets on restart, so after starting the app: put the robot in the pose you want (or let the action's own pose step do it) and press the button.

•	
      Velocity/driving commands are only honoured while the robot is standing; while sitting they are ignored silently.

1.3 Adding or editing a command (config.json)

Each button is one entry under "actions". Example:

"twist_jump": {

"enabled": true,

"note": "twist jump (0x2101020D - standing, moves dog)",

"posture": "stand",

"force_posture": true,

"replay": "slow",

"group": "action",

"icon": "🌪️",

"label": "Twist jump",

"frame": { "code": "0x2101020D", "type": 0, "value": 0 }

}

•	
      frame.code — the 32-bit command (hex). type 0 = simple frame; value — integer parameter (velocity frames use payload instead).

•	
      posture / force_posture — pose handling as above. Omit for actions valid in any pose.

•	
      replay: "slow" — sends the frame three times at ~1 Hz (used for poses/rolls). "once": true — a single frame (used for mode/gait switches). Default — short burst.

•	
      group — which block the button appears in (recovery / action / mode / gait). icon + label — button face; note — tooltip.

•	
      After editing: restart the console (or reload the page) — buttons appear automatically. Disable a button with "enabled": false.

1.4 Finding the right code for a new command

1	
      Probe: Advanced → raw frames — send a frame once and watch the robot.

2	
      Capture from the official remote: run tools/retroid_listen.py on the console computer, point the RETROID controlapp at <console IP>:12121, press the button there, and read the exact frame.

3	
      Safety: test new poses on a soft surface with clear space; keep the physical E-stop in reach. E-STOP in the console cancels any action mid-play and blocks all commands until ⟲ Return.

2. Two-joystick movement

2.1 Layout (exactly as in the app)

•	
      Left pad — labelled FWD · turn ← → · BACK: up/down = forward/back, left/right = turn.

•	
      Right pad — labelled FWD · slide ← → · BACK: left/right = slide sideways, up/down = forward/back. If both pads push forward, the stronger one applies.

•	
      Diagonals combine axes (forward + turn, forward + slide, etc.).

•	
      Above the pads: ↤ Slide left and ↦ Slide right buttons — hold either to side-walk without using the right pad.

•	
      The readout under the pads shows the live values: fwd · side · turn.

2.2 Behaviour

•	
      Both pads are spring-centred: the knob follows the pointer while held, and on release it snaps back to the centre — which stops the robot and sends the brake frames immediately.

•	
      Drive input is sent live at ~60 Hz while a pad is held (zero-latency updates on press and move).

•	
      Turning and sliding are also available from the buttons above the pad, so the robot can be driven pad-only, buttons-only, or a mix.

2.3 Stopping

•	
      Release the pad (or return it to the centre) → stop.

•	
      STOP button → official brake frames (0x21010C0B with values 0 and 1).

•	
      E-STOP → latched: all commands are blocked until ⟲ Return is pressed (also cancels any action mid-play).

•	
      Switching browser tab, minimising the window or losing focus also stops the robot (safety).

2.4 Tuning (Advanced panel)

•	
      Stick curve — Linear / Soft (default, 1.25) / Soft+: higher value = finer control near the centre, which is what most people want for delicate moves.

•	
      Speed scale — 50 % / 75 % / 100 % (default) / 150 %: multiplies the configured maxima below.

•	
      Configured maxima — forward/back 0.5 m/s, lateral slide 0.4 m/s, turn 0.8 rad/s.

•	
      Forward sign / Turn sign / Lateral sign — flips the axis if a direction is reversed on your unit; changes apply immediately.

•	
      Joystick mode — Velocity frames (proven) sends the standard velocity frames (0x140 / 0x145 / 0x141); Retroid / official SDK instead sends the same 42-byte frames as the physical remote (UDP 12121) and lets the robot's own controller interpret the sticks.

2.5 Keyboard (alternative to the pads)

•	
      W / S (or ↑ / ↓) — forward / back · A / D (or ← / →) — turn.

•	
      Space — hold-to E-stop.

•	
      Releasing the keys stops the robot; keys and pads can be mixed.

3. Camera setup

3.1 How the video path works

•	
      The console asks the robot to start its camera services (command 0x21012109, value 0x40, to UDP 43899) and reads the stream at rtsp://<robot IP>:8554/test, re-publishing it to the browser as MJPEG.

•	
      Feed opens automatically after connecting and is kept open by a watchdog that retries every ~8 s (re-sending the start command, reconnecting the stream).

3.2 If the robot publishes nothing (RTSP 404 / no video)

1	
      Confirm the robot is on and reachable: ping 192.168.2.1 (robot Wi-Fi) or 192.168.1.120 (LAN).

2	
      Check the RTSP port is open and whether the stream name exists:
nc -z -w2 192.168.2.1 8554
ffprobe -v error -rtsp_transport tcp -show_entries format=format_name -of csv=p=0 rtsp://192.168.2.1:8554/test
A working stream prints rtsp; "404 Not Found" means the camera service on the robot is not publishing.

3	
      If the port is open but the stream is absent, the robot's camera service must be started on the robot (its own service/tooling); power-cycling the robot usually restores it.

3.3 Tuning quality for a stable feed (Advanced → Camera stream)

•	
      FPS — 4–6 fps is plenty for driving (lower = lighter).

•	
      Scale — e.g. 480:-1 (default, light) or 640:-1 (sharper, heavier); 320:-1 for the lowest bandwidth.

•	
      Quality — MJPEG -q:v: 2 = best/largest, 18 = smallest; default 11 balances both.

•	
      Apply & restart camera — applies immediately; Camera off stops the feed.

•	
      Rule of thumb: if the picture stutters on the robot's Wi-Fi, lower FPS first, then scale, then raise the quality number.

Ports: commands 43893 (UDP) · telemetry 43897 (UDP) · camera control 43899 (UDP) · RTSP 8554 (TCP, /test) · remote 12121 (UDP) · console 8123 (HTTP).