Lite3 Pilot — joystick movement logic & full command catalog

Complete, implementation-accurate reference for the web console (port 8123) and the Python SDK. All values are the live defaults in config.json.

1. Joystick movement logic

1.1 Inputs

Input

Mapping

Left pad (labels FWD / turn / BACK)

up/down = forward/back (vx) · left/right = turn (wz)

Right pad (labels FWD / slide / BACK)

left/right = slide sideways (vy) · up/down = forward/back — the stronger of the two pads wins for vx

↤ Slide left / ↦ Slide right buttons

hold to side-walk at 0.4 m/s × speed-scale (sends lateral velocity; in Retroid mode the side axis of the 42-byte frame)

Keyboard

W/S or ↑/↓ = forward/back · A/D or ←/→ = turn · Space (hold) = E-STOP

Pads and keys can be mixed; diagonals combine axes (e.g. forward + slide).

1.2 Per-axis processing (identical for both pads)

1	
      Vector from pointer: each pad returns x/y in −1…+1 relative to its centre.

2	
      Deadzone: magnitude below 0.08 = neutral (no command).

3	
      Normalisation: the magnitude past the deadzone is rescaled so small pushes still count: s = (m − 0.08) / (1 − 0.08), clamped to 1.

4	
      Stick curve: s = s ^ curve when curve > 1 (default 1.25, “Soft”) — finer control near the centre.

5	
      Speed scale: multiply by the scale setting (50 / 75 / 100 / 150 %).

6	
      Axis assignment: vx = −y × s × scale · wz = x × s × scale (left pad) · vy = x × s × scale (right pad).

7	
      Reverse-steer mirror: when reversing (vx < 0) the steering sign is flipped, so “left” still turns the robot left while backing up.

8	
      Clamps (server side): vx ≤ 0.5 m/s · vy ≤ 0.4 m/s · wz ≤ 0.8 rad/s · per-axis sign (Forward / Turn / Lateral) applied on send.

1.3 Sending

•	
      Commands are produced by a 60 ms loop; additionally a drive update fires immediately on press and on every move event (zero-latency response).

•	
      The engine streams at up to 50 Hz (rate_hz) while the request is fresh, keeping the robot in auto mode (0x21010C03 interleaved) with a heartbeat (0x21040001) every 300 ms.

•	
      Tail: the last velocity is kept alive for 300 ms after the last input, then zeroed; when a velocity stream ends the engine also fires the official brake frames (0x21010C0B, values 0 and 1) for ~0.35 s so the robot cannot coast.

1.4 Release & stopping

•	
      Pads are spring-centred: on release the knob animates home (160 ms) and the robot stops — a stop command is sent immediately, not on the next tick.

•	
      Release anywhere (window-level capture), losing window focus, hiding the tab, STOP, or E-STOP all stop the robot.

•	
      E-STOP latches: every command is refused and stop frames keep being sent until ⟲ Return; a latched E-stop also cancels a running action.

1.5 Drive modes (Advanced → Joystick mode)

•	
      Velocity frames (proven) — sends 0x140 (x), 0x145 (y), 0x141 (yaw), each type 1 with one f64 payload (value field 8), plus the auto-mode frame.

•	
      Retroid / official SDK — sends the same 42-byte frames as the physical remote to UDP 12121: header 0x55 0x66, ctrl, data_len 32, seq, id 1, checksum (byte-sum of payload); payload = 10 × u16 buttons + 4 × i16 axes (left_x, left_y, right_x, right_y) + 2 × u16 axis buttons; range ±1000. Mapping: left_y = forward, right_x = yaw (sign configurable), left_x = slide. The robot's own controller then interprets the sticks.

1.6 Velocity semantics

•	
      Velocity commands are honoured only while the robot is standing (in auto mode); while sitting/lying they are ignored silently.

•	
      UDP gives no acknowledgement; confirm motion visually or via telemetry.

2. Command catalog (identical to the app buttons)

2.1 Built-in top-row buttons

Button

Command

Code

Behaviour

🐕 STAND

posture toggle

0x21010202

One code for both poses: from sitting it stands; if already standing it sits (state-tracked, skipped when already there)

🪑 SIT

posture toggle

0x21010202

Same toggle, aimed at the sitting pose

👋 HELLO

hello pose

0x21010506

Sits first if needed, then replays the pose 3× at ~1 Hz (verified on the robot)

🕺 DANCE

dance / moonwalk

0x2101030C

Standing pose; verified

🔄 TURN OVER

turn over

0x21010205

Rolls the robot onto its back; forces the sitting pose first; slow replay

2.2 Recovery block (from upside down)

Button

Command

Code

Behaviour

↺ Recover left / ↻ Recover right

stand toggle used as recovery

0x21010202

Replays the stand toggle at 1 Hz to bring the robot back to its feet (proved on the robot)

2.3 Poses & actions

Button

Command name

Code

Required pose

🌀 Twist

twist

0x21010204

standing

🌪️ Twist jump

twist_jump

0x2101020D

standing (forced)

🧘 Backflip

backflip

0x21010502

sitting (forced)

🦘 Long jump

long_jump

0x2101050B

sitting (forced)

👋 Hello / 🕺 Dance / 🔄 Turn over

hello / dance / turn_over

0x21010506 / 0x2101030C / 0x21010205

sit / stand / sit (forced)

2.4 Modes

Button

Command name

Code

Behaviour

🎮 Manual mode

mode_manual

0x21010C02

single-shot switch (remote/manual mode)

🕹️ Move mode

mode_move

0x21010D06

single-shot switch (move mode)

2.5 Gaits

Button

Command name

Code

Behaviour

🐢 Slow

gait_slow

0x21010300

single-shot gait switch

🚶 Medium

gait_medium

0x21010307

single-shot gait switch

⚡ Fast

gait_fast

0x21010303

single-shot gait switch

🐛 Crawl

gait_crawl

0x21010406

single-shot gait switch

2.6 Execution rules that apply to every command

•	
      Posture handling: an action declares its required pose; the console re-poses only when the tracked state is known to differ, or always when force_posture is set (rolls, jumps, hello). With an unknown state and no force, the action is sent directly (press STAND/SIT first if the robot is in the other pose).

•	
      Replay: poses/rolls are re-sent 3× at ~1 Hz (“slow replay”); modes/gaits are single-shot; other frames use a short burst.

•	
      Cancellation: E-STOP aborts any action mid-play.

•	
      Safety: keep the area clear and the physical E-stop within reach for any moving command.

Sources: DeepRobotics repositories and manuals, community protocol tables, and live verification on the robot (hello, dance, stand/sit, recovery, velocity driving, camera). Codes marked “forced/logged” but not yet verified on this unit should be confirmed on a stand before regular use.