DeepRobotics Jueying Lite3 — Operating from a web browser

No software installation is required on the operating device (phone, tablet or laptop). The device only needs to be on the same network as the console computer: the robot's Wi-Fi (192.168.2.x) or the same LAN (192.168.1.x).

1. Requirements

•	
      The console computer is powered on, on the robot's Wi-Fi or on the same LAN, and the Lite3 Pilot console is running (it serves the web interface on port 8123).

•	
      Your device (phone/tablet/laptop) is:

◦	
      on the robot's Wi-Fi (the hotspot from the robot's QR label), or

◦	
      on the same LAN as the console computer.

•	
      No web server, no VPN, no internet access is needed — everything is local.

2. Find the console address

1	
      On the console computer, the address to use is its own IP on the network that your device shares:

◦	
      on the robot's Wi-Fi: e.g. 192.168.2.22

◦	
      on the same LAN: e.g. 192.168.1.210

2	
      Console URL format: http://<console-computer-IP>:8123
        Example: http://192.168.2.22:8123

3. Open and connect

1	
      Open that URL in the browser on your device.

2	
      Check the robot IP shown under Advanced → Robot IP: it must be the motion host of the network you are on:

◦	
      robot Wi-Fi: 192.168.2.1

◦	
      ethernet/LAN: 192.168.1.120

3	
      Press 🔌 Connect. Wait for ✅ Connected (the status line also shows the ping result).

4	
      The camera pane opens automatically (when the robot's camera service is running).

5	
      Operating controls: 🐕 STAND / 🪑 SIT, the action buttons (dance, hello, flips…), the two joysticks (left: forward/back + turn, right: slide), STOP and E-STOP.

4. Before you move the robot

•	
      Clear the area (about 5 m) and keep the physical E-stop within reach — it always overrides the software.

•	
      The robot must be standing: driving commands are ignored while it is sitting or lying down.

•	
      If the header shows E-STOP latched, press ⟲ Return to re-arm. While latched, no command is sent.

•	
      Both joysticks are spring-centred: release = stop. Holding a position keeps driving until released.

5. Troubleshooting

•	
      The page does not open: your device is not on the same network as the console computer, or the IP is wrong. Re-join the robot Wi-Fi / LAN and re-check the IP.

•	
      Page opens but not connected: verify Advanced → Robot IP matches the network (2.1 on robot Wi-Fi, 1.120 on LAN), and that the robot is powered and awake.

•	
      Buttons are greyed until connected: expected. They enable automatically on connect.

•	
      Commands seem to do nothing: robot not standing, or software E-stop latched (press ⟲ Return). Note that UDP gives no acknowledgement — confirm motion visually.

•	
      No camera: the robot's camera service must be running on the robot. The console retries automatically every ~8 s.

•	
      No battery / IMU readout: requires a one-time setting on the robot (telemetry target in ~/jy_exe/conf/network.toml). Driving works without it.

Security note: the console is a local service without authentication — keep it on trusted, local networks and close the browser session when finished.